# DAC Circuit Simulation Using R-2R Ladder Network

ENGS152 Circuits Lab

7 February 2026

# Schematic

# Introduction

DAC (Digital to Analog Converter) is a special circuit that allows you to convert digital signals to analog. VA to VD pins are binary inputs of the 4-bit DAC. The input pins are either connected to 5V, either to the ground, correspondingly 0 and 1. The output voltage is determined by the combination of the input voltages.

Vout=Vref×D2n

The resistance values of 10 kΩ and 20 kΩ were chosen randomly, however the one to two ration was kept. One can estimate the maximum current that could flow in the circuit, assuming that only MSB is connected to 5 V, and other terminals are connected to the GND. This creates the combination of the lowest resistance in the circuit and allows more current to flow in the circuit.

Im=Vref2R = 0.25 mA, for Vref=5V and R = 10 kΩ.